Simple ATM Machine

Team Members:

Alaa Ibrahim

Bassel Yasser

Sharpel Malek

Sherif Khadr

Contents

INTRODUCTION	1

High Level Design	1

01)	Layered Architecture	1

02)	Modules Description	2

03)	Drivers’ Documentation	3

MCAL Layer	3

	DIO	3

	Timer 0	4

	EXTINT:	5

	I2C:	6

	UART:	9

	SPI:	10

HAL Layer:	11

	Keypad	11

	HTimer:	12

	HLCD	13

	Buzzer	14

	Button	14

	EEPROM	15

Application Layer:	16

Low Level Design	17

MCAL Layer:	17

	DIO	17

	Timer:	20

	EXTINT:	22

	UART:	23

	SPI:	24

	I2C:	25

HAL Layer	29

	HTimer0	29

	LCD	30

	Keypad	37

	Buzzer	40

	HEXTINT:	40

	Button:	41

Application Layer:	42

Figure 1: Project Layered Architecture	1

Figure 2 DIO_s8SETPinDir Flow Chart	12

Figure 3 DIO_s8SETPinVal Flow chart	13

Figure 4 DIO_s8GETPinVal Flow Chart	14

Figure 5 TIM0_Init Flow Chart	15

Figure 6  TIM0_Start Flow Chart	15

Figure 7 TIM0_Stop Flow Chart	15

Figure 8 TIM0 remaining Flow Charts	16

Figure 9 ADC_Init Flow Chart	17

Figure 10 ADC_SetChannel Flow Chart	18

Figure 11 ADC_Deinit Flow Chart	18

Figure 12 ADC_StartConversion Flow Chart	18

Figure 13 ADC_GetConversionResult Flow Chart	19

Figure 14 ADC_Conversion Flow Chart	20

Figure 15 HTIM0_SyncDelay Flow Chart	21

Figure 16 HTIM0_AsyncDelay and EndDelay	21

Figure 17 HLCD_vidInit Flow Chart	22

Figure 18 HLCD_vidWritecmd Flow Chart	23

Figure 19 HLCD_vidWriteChar Flow Chart	23

Figure 20 HLCD_ClrDisplay Flow Chart	24

Figure 21 HLCD_gotoXY Flow Chart	25

Figure 22 HLCD_WriteString Flow Chart	26

Figure 23 HLCD_WriteInt Flow Chart	27

Figure 24 HLCD_vidCreatCustomChar Flow Chart	28

Figure 25 KEYPAD_Init Flow Chart	29

Figure 26 KEYPAD_CheckRx Flow Chart	30

Figure 27 GetButton Flow Chart	31

Figure 28 Buzzer Init & SetState Flow Charts	32

Figure 29 Temp Sensor Init & ReadValue Flow Charts	32

Figure 30 App. APIs Flow Charts	33

Figure 31 APP_Start & App States Flow Charts	34

Figure 32 Cont. App States Flow Charts	35

# INTRODUCTION

# High Level Design

## Layered Architecture

ATM MACHINE:

Figure 1: Project Layered Architecture

CARD

## Modules Description

MCAL Layer:

- DIO: For controlling GPIO pins

- Timer: Provides an interface with timer 0 low-level capabilities.

- EXTINT:

- USART:

- SPI:

- I2C:

HAL Layer:

- Keypad: Deal with a set of buttons arranged in a block. The 3 x 3 matrix keypad usually is used as input in a project

- LCD: Use for display data

- Buzzer: Simple module to control a buzzer.

- HTimer: Provides high-level functions using the lower-level timer 0 module capabilities.

- EEPROM:

- Button:

- Buzzer:

Service Layer:

- STD_Types: Contains all the standard types used by all the layers.

- BIT_Math: Provides bit-wise operations.

- Vect_table: Contains all interrupt vectors and provides macros for dealing with general interrupt.

Application Layer:

Contains the main logic of the project.

## Drivers’ Documentation

### MCAL Layer

#### DIO

/*

* AUTHOR		: Bassel Yasser

* Function		: DIO_s8SETPinDir

* Description	: Set Pin Direction

* Arguments	:

* 					- enPinCopy {DIO_PINA_0...., DIO_PIND_7}

* 					- enPortDir {INPUT , OUTPUT}

* Return		:   Sint8_t

*/

Sint8_t DIO_s8SETPinDir (enu_pin enPinCopy, enu_dir enPortDir)

/*

* AUTHOR		: Bassel Yasser

* Function		: DIO_s8SETPinVal

* Description	: Set Pin Value

* Arguments	:

* 					- enPinCopy {DIO_PINA_0...., DIO_PIND_7}

* 					- enPortDir {HIGH , LOW}

* Return		:   Sint8_t

*/

Sint8_t DIO_s8SETPinVal (enu_pin enPinCopy, enu_val enPortVal)

/*

* AUTHOR		: Bassel Yasser

* Function		: DIO_s8GETPinVal

* Description	: Set Pin Value

* Arguments	:

* 					- enPinCopy {DIO_PINA_0...., DIO_PIND_7}

* 					- pu8Val address of variable that u want to save value on it

* Return		:   Sint8_t

*/

Sint8_t DIO_s8GETPinVal (enu_pin enPinCopy, Uint8_t* pu8Val)

#### Timer 0

/**

* \brief Initialize the timer with given mode

* \param u8_a_Mode

* \return en_TIMErrorState_t

*/

en_TIMErrorState_t TIM0_voidInit(en_TIMMode_t u8_a_Mode);

/**

* \brief Start the timer clock after prescaling it with given value

* \param u8_a_prescaler

* \return en_TIMErrorState_t

*/

en_TIMErrorState_t TIM0_Start(en_TIM_CLK_SELECT_t u8_a_prescaler);

/**

* \brief Function to stop timer 0

* \return void

*/

void TIM0_Stop();

/**

* \brief Set the timer to start from a certain value

* \param u8_a_FlagValue The value to start the timer from

* \return void

*/

void TIM0_SetValue(Uchar8_t u8_a_startValue);

/**

* \brief Function to get the value of the overflow flag of timer 0

* \param u8_a_FlagValue reference to a variable to store flag value *

* \return en_TIMErrorState_t

*/

en_TIMErrorState_t TIM0_GetOVF(Uchar8_t* u8_a_FlagValue);

/**

* \brief Function to clear timer 0 overflow flag

* \return void

*/

void TIM0_ClearOVF(void);

/**

* \brief Function to get the timer state (running/stopped)

* \param u8_a_State reference to a variable to store timer state

* \return en_TIMErrorState_t

*/

en_TIMErrorState_t TIM0_GetState(en_TIMState_t* u8_a_State);

/**

* \brief Function to set a function to call when the timer0

*	      Overflow Interrupt is triggered

* \param pv_a_CallbackFn reference to the function to call

* \return en_TIMErrorState_t

*/

#### EXTINT:

#### I2C:

/***********************************************************************************/

/*			01- i2c_init_master()									*/

/* ----------------------------------------------------------------------------------

* @func   : I2C master Initialization

* @in     : void

* @out    : void

************************************************************************************/

void i2c_init_master(void);

/***********************************************************************************/

/*			02- i2c_init_slaver()									*/

/* ----------------------------------------------------------------------------------

* @func   : I2C slave Initialization

* @in     : void

* @out    : void

************************************************************************************/

void i2c_init_slave (void);

/***********************************************************************************/

/*			03- i2c_start()									*/

/* ----------------------------------------------------------------------------------

* @func   : Send start condition

* @in     : void

* @out    : void

************************************************************************************/

void i2c_start(void);

/***********************************************************************************/

/*			04- i2c_repeated_start()									*/

/* ----------------------------------------------------------------------------------

* @func   : Send repeated start condition

* @in     : void

* @out    : void

************************************************************************************/

void i2c_repeated_start(void);

/***********************************************************************************/

/*			05- i2c_send_slave_address_with_write_req()									*/

/* ----------------------------------------------------------------------------------

* @func   		: send write request from master to slave

* @in[1]     	: slave_address

* 					- Slave address that you want to write on it

* @out       	: void

************************************************************************************/

void i2c_send_slave_address_with_write_req(Uint8_t slave_address);

/***********************************************************************************/

/*			06- i2c_send_slave_address_with_read_req()									*/

/* ----------------------------------------------------------------------------------

* @func   		: send read request from master to slave

* @in[1]     	: slave_address

* 					- Slave address that you want to read from it

* @out       	: void

************************************************************************************/

void i2c_send_slave_address_with_read_req(Uint8_t slave_address);

/***********************************************************************************/

/*			07- i2c_write_byte()									*/

/* ----------------------------------------------------------------------------------

* @func   		: Write data to slave

* @in[1]     	: byte

* 					- write data that u need to send

* @out       	: void

************************************************************************************/

void i2c_write_byte(Uint8_t byte);

/***********************************************************************************/

/*			08- i2c_read_byte()									*/

/* ----------------------------------------------------------------------------------

* @func   		: read data from slave

* @in	     	: void

* @out       	: returned data

************************************************************************************/

Uint8_t i2c_read_byte(void);

/***********************************************************************************/

/*			09- i2c_stop()									*/

/* ----------------------------------------------------------------------------------

* @func   		: Send Stop condition

* @in	     	: void

* @out       	: void

************************************************************************************/

void i2c_stopvoid);

#### UART:

#### SPI:

### HAL Layer:

#### Keypad

// Macros

#define R1     DIO_PINC_2

#define R2     DIO_PINC_3

#define R3     DIO_PINC_4

#define C1     DIO_PINC_5

#define C2     DIO_PINC_6

#define C3     DIO_PINC_7

// user defined datatypes

typedef enum EN_KEYPAD_BTNS

{

KEY_INCREAMENT=0,

KEY_DECREAMENT,

KEY_SET,

KEY_ADJUST,

KEY_RESET,

KEY_6,

KEY_7,

KEY_8,

KEY_9,

KEY_NOTHING

}EN_KEYPAD_BTNS;

// functions prototypes

/*************************************************************************************************************

*

Name : KEYPAD_init()

Description : This Function Initializes keypad pins (Rows are outputs & Columns are inputs).

ARGS : void

return : void

************************************************************************************************************/

void KEYPAD_init(void);

/*************************************************************************************************************

*

Name : KEYPAD_GetButton

Description : This Function loops over other three functions (Checks (R1,R2,R3)).

ARGS : void

return : the pressed key or Nothing pressed

************************************************************************************************************/

EN_KEYPAD_BTNS KEYPAD_GetButton(void);

/*************************************************************************************************************

*

Name : KEYPAD_checkR1 ,  KEYPAD_checkR2, KEYPAD_checkR3

Description : functions are checking the entire row if it pressed or not.

ARGS : void

return : the pressed key or Nothing pressed

************************************************************************************************************/

EN_KEYPAD_BTNS KEYPAD_checkR1(void);

EN_KEYPAD_BTNS KEYPAD_checkR2(void);

EN_KEYPAD_BTNS KEYPAD_checkR3(void);

#### HTimer:

/**

* \brief Generate Synchronous delay (busy waiting)*

* \param Copy_delayTime Desired delay

* \param Copy_timeUnit Time units (Seconds, mSeconds, uSeconds)

*

* \return en_HTIMErrorState_t

*/

en_HTIMErrorState_t TIM0_SyncDelay(Uint32_t u32_a_delay, en_timeUnits_t u8_a_timeUnit);

/**

* \brief Generates delay asynchronously

* \param u32_a_delay desired delay

* \param u8_a_timeUnit delay time units

* \param Copy_pvCallbackFn function to call when delay is complete

*

* \return en_TIMErrorState_t

*/

en_HTIMErrorState_t TIM0_AsyncDelay(Uint32_t u32_a_delay, en_timeUnits_t u8_a_timeUnit, void (*Copy_pvCallbackFn)(void));

/**

* \brief Function to end a delay asynchronously

* To Stop Async Delay: No Restrictions

* To Stop Sync Delay: should only be called in an ISR/Callback function

*

* \return void

*/

void TIM0_AsyncEndDelay();

#### HLCD

/*

* function		: HLCD_vidInit

* description 	: func to set LCD initialization

* input param 	: void

* return		: void

* */

void HLCD_vidInit(void)

/*

* function		: HLCD_vidWritecmd

* description 	: func to configure some commands on lcd

* input param 	:

* 					u8commandCopy --> take lcd cmd instructions from instruction table <https://components101.com/sites/default/files/component_datasheet/16x2%20LCD%20Datasheet.pdf>

* return		: void

* */

void HLCD_vidWritecmd(Uint8_t u8commandCopy)

/*

* function		: HLCD_vidWriteChar

* description 	: func to write char on lcd

* input param 	: u8CharCopy -> take ascii code of char   or   char address on CGROM

* return		: void

* */

void HLCD_vidWriteChar(Uint8_t u8CharCopy)

/*

* function		: HLCD_ClrDisplay

* description 	: func to clear anything on lcd

* input param 	: void

* return		: void

* */

void HLCD_ClrDisplay(void)

/*

* function		: HLCD_gotoXY

* description 	: func to determine position which char print at this position on lcd  ### NOTE : (2rows x 16coloms)

* input param 	:

* 				  row -> take row number 0 or 1

* 				  pos -> take colom number from 0 ~ 16

* return		: void

* */

void HLCD_gotoXY(Uint8_t row, Uint8_t pos)

/*

* function		: HLCD_WriteString

* description 	: func to write string on lcd

* input param 	: str --> which take string as argument

* return		: void

* */

void HLCD_WriteString(Uint8_t* str)

/*

* function		: HLCD_WriteInt

* description 	: func to write integer number on lcd

* input param 	: number --> which take number as argument

* return		: void

* */

void HLCD_WriteInt(Uint32_t number)

/*

* function		: HLCD_vidCreatCustomChar

* description 	: func to store new patterm on CGRAM

* input param 	:

* 				  pu8custom  -> take pointer to array which having LCD Custom Character Generated data ### take only 8 characters

* 				  u8Location -> determine location on CGRAM [0 ~ 8]

* return		: void

* */

void HLCD_vidCreatCustomChar(Uint8_t* pu8custom, Uint8_t u8Location)

#### Buzzer

/**

* \brief Initialize buzzer pin as output

* \param pst_a_buzzer reference to buzzer

* \return void

*/

void BUZ_Init(st_Buzzer_t* pst_a_buzzer);

/**

* \brief Turn the buzzer on/off

* \param pst_a_buzzer reference to buzzer

* \param u16_a_state BUZ_ON (or) BUZ_OFF

* \return en_BuzzerErrorState_t

*/

en_BuzzerErrorState_t BUZ_SetState(st_Buzzer_t* pst_a_buzzer, en_BuzzerState_t en_a_state);

#### Button

/*

* AUTHOR			: Bassel Yasser Mahmoud

* FUNCTION			: HButton_Init

* DESCRIPTION		: Initialize specified pin as input and pull up

* RETURN			: enu_buttonError_t {BUTTON_NOK, BUTTON_OK}

*/

enu_buttonError_t HButton_Init(enu_pin en_pinx);

/*

* AUTHOR			: Bassel Yasser Mahmoud

* FUNCTION			: HButton_ExtIntInit

* DESCRIPTION		: Initialize specified as pull up for external interrupt

* RETURN			: enu_buttonError_t {BUTTON_NOK, BUTTON_OK}

*/

enu_buttonError_t HButton_ExtIntInit(enu_pin en_pinx);

/*

* AUTHOR			: Bassel Yasser Mahmoud

* FUNCTION			: HButton_getPinVal

* DESCRIPTION		: Get pin status if it is high or low

* RETURN			: enu_buttonError_t {BUTTON_NOK, BUTTON_OK}

*/

enu_buttonError_t HButton_getPinVal(enu_pin en_pinx,  Uint8_t* pu8_refVal );

#### EEPROM

### Application Layer:

/**

* \brief Initialize all modules and execute welcome routine

*

* \param

*

* \return void

*/

void APP_Init(void);

/**

* \brief Application main logic

*

* \param

*

* \return void

*/

void APP_Start(void);

/**

* \brief Initialize temperature adjustment process

*

* \param

*

* \return void

*/

static void APP_adjustInit(void);

/**

* \brief timeout callback function

*

* \return void

*/

void timeout(void);

# Low Level Design

### MCAL Layer:

#### DIO

Sint8_t DIO_s8SETPinDir (enu_pin enPinCopy, enu_dir enPortDir)

Figure 2 DIO_s8SETPinDir Flow Chart

Sint8_t DIO_s8SETPinVal (enu_pin enPinCopy, enu_val enPortVal)

Figure 3 DIO_s8SETPinVal Flow chart

Sint8_t DIO_s8GETPinVal (enu_pin enPinCopy, Uint8_t* pu8Val)

Figure 4 DIO_s8GETPinVal Flow Chart

#### Timer:

TIM0_Init

Figure 5 TIM0_Init Flow Chart

TIM0_Start

Figure 6  TIM0_Start Flow Chart

TIM0_Stop

Figure 7 TIM0_Stop Flow Chart

Figure 8 TIM0 remaining Flow Charts

#### EXTINT:

#### UART:

#### SPI:

#### I2C:

void i2c_init_master(void);

void i2c_start(void);

void i2c_repeated_start(void);

void i2c_send_slave_address_with_write_req(Uint8_t slave_address);

void i2c_send_slave_address_with_read_req(Uint8_t slave_address);

void i2c_write_byte(Uint8_t byte);

Uint8_t i2c_read_byte(void);

void i2c_stop(void);

### HAL Layer

#### HTimer0

HTIM0_SyncDelay

Figure 15 HTIM0_SyncDelay Flow Chart

Figure 16 HTIM0_AsyncDelay and EndDelay

#### LCD

Figure 17 HLCD_vidInit Flow Chart

Figure 17 HLCD_vidInit Flow Chart

void HLCD_vidInit(void)

void HLCD_vidWritecmd(Uint8_t u8commandCopy)

Figure 18 HLCD_vidWritecmd Flow Chart

Figure 18 HLCD_vidWritecmd Flow Chart

void HLCD_vidWriteChar(Uint8_t u8CharCopy)

Figure 19 HLCD_vidWriteChar Flow Chart

Figure 19 HLCD_vidWriteChar Flow Chart

Figure 20 HLCD_ClrDisplay Flow Chart

Figure 20 HLCD_ClrDisplay Flow Chart

void HLCD_ClrDisplay(void)

void HLCD_gotoXY(Uint8_t row, Uint8_t pos)

Figure 21 HLCD_gotoXY Flow Chart

void HLCD_WriteString(Uint8_t* str)

Figure 22 HLCD_WriteString Flow Chart

void HLCD_WriteInt(Uint32_t number)

Figure 23 HLCD_WriteInt Flow Chart

void HLCD_vidCreatCustomChar(Uint8_t* pu8custom, Uint8_t u8Location)

Figure 24 HLCD_vidCreatCustomChar Flow Chart

#### Keypad

Figure 25 KEYPAD_Init Flow Chart

Figure 26 KEYPAD_CheckRx Flow Chart

Figure 27 GetButton Flow Chart

#### Buzzer

BUZ_SetState

BUZ_SetState

Figure 28 Buzzer Init & SetState Flow Charts

Figure 28 Buzzer Init & SetState Flow Charts

BUZ_Init

BUZ_Init

#### HEXTINT:

#### Button:

enu_buttonError_t HButton_Init(enu_pin en_pinx);

enu_buttonError_t HButton_ExtIntInit(enu_pin en_pinx);

enu_buttonError_t HButton_getPinVal(enu_pin en_pinx,  Uint8_t* pu8_refVal );

### Application Layer:

Figure 30 App. APIs Flow Charts

Figure 31 APP_Start & App States Flow Charts

Figure 32 Cont. App States Flow Charts