Simple ATM Machine

Team Members:

Alaa Ibrahim

Bassel Yasser

Sharpel Malek

Sherif Khadr

Contents

INTRODUCTION	1

High Level Design	1

01)	Layered Architecture	1

02)	Modules Description	2

03)	Drivers’ Documentation	3

MCAL Layer	3

	DIO	3

	Timer 0	4

	EXTINT:	5

	I2C:	6

	UART:	9

	SPI:	10

HAL Layer:	11

	Keypad	11

	HTimer:	12

	HLCD	13

	Buzzer	14

	Button	14

	HEXT_INT	15

	HSPI	15

	EEPROM:	16

Card_Database Layer (CARD MCU)	18

ATM_MODULE Layer (ATM MCU)	19

	ATM:	19

	Database_check:	20

Application Layer:	21

	Card MCU	21

	ATM MCU	21

Low Level Design	22

MCAL Layer:	22

	DIO	22

	Timer:	25

	EXTINT:	27

	UART:	30

	SPI:	34

	I2C:	35

HAL Layer	39

	HTimer0	39

	HSPI	40

	LCD	41

	Keypad	48

	Buzzer	51

	HEXTINT:	51

	Button:	52

	EEPROM	53

Card_Database Layer (CARD MCU)	58

	Database_check:	60

	ATM Module	64

Application Layer:	67

	Card App	67

	ATM App	67

Figure 1: Project Layered Architecture	1

Figure 2 DIO_s8SETPinDir Flow Chart	20

Figure 3 DIO_s8SETPinVal Flow chart	21

Figure 4 DIO_s8GETPinVal Flow Chart	22

Figure 5 TIM0_Init Flow Chart	23

Figure 6  TIM0_Start Flow Chart	23

Figure 7 TIM0_Stop Flow Chart	23

Figure 8 TIM0 remaining Flow Charts	24

Figure 15 HTIM0_SyncDelay Flow Chart	40

Figure 16 HTIM0_AsyncDelay and EndDelay	40

Figure 17 HLCD_vidInit Flow Chart	42

Figure 18 HLCD_vidWritecmd Flow Chart	43

Figure 19 HLCD_vidWriteChar Flow Chart	43

Figure 20 HLCD_ClrDisplay Flow Chart	44

Figure 21 HLCD_gotoXY Flow Chart	45

Figure 22 HLCD_WriteString Flow Chart	46

Figure 23 HLCD_WriteInt Flow Chart	47

Figure 24 HLCD_vidCreatCustomChar Flow Chart	48

Figure 25 KEYPAD_Init Flow Chart	49

Figure 26 KEYPAD_CheckRx Flow Chart	50

Figure 27 GetButton Flow Chart	51

Figure 28 Buzzer Init & SetState Flow Charts	52

Figure 30 App. APIs Flow Charts	54

Figure 31 APP_Start & App States Flow Charts	55

Figure 32 Cont. App States Flow Charts	56

# INTRODUCTION

# High Level Design

## Layered Architecture

ATM MACHINE:

CARD MCU

## Modules Description

MCAL Layer:

- DIO: For controlling GPIO pins

- Timer: Provides an interface with timer 0 low-level capabilities.

- EXTINT: it controls three external hardware interrupts on pins PD2, PD3, and PB2 which are referred to as INT0, INT1, and INT2 respectively

- USART: Enables MCU to communicate using serial protocols.

- SPI: Provides interface with SPI’s low-level functionalities.

- I2C: Handles the i2c address oriented, multi-master, multi-slave communication.

HAL Layer:

- Keypad: Deal with a set of buttons arranged in a block. The 3 x 3 matrix keypad usually is used as input in a project

- LCD: Use for display data

- Buzzer: Simple module to control a buzzer.

- HTimer: Provides high-level functions using the lower-level timer 0 module capabilities.

- H_EXT_INT: Provides middle ware layer between app and external interrupt in MCAL.

- HUSART: Provides middle ware layer between app and UART in MCAL.

- HSPI: Provides high-level functionalities for using SPI communication.

- EEPROM: Enables MCU to interface with external EEPROM using i2c.

- Button: Interface with button to get its state (Enter/Zero).

- Buzzer: Enables MCU to control any given buzzer.

Service Layer:

- STD_Types: Contains all the standard types used by all the layers.

- BIT_Math: Provides bit-wise operations.

- Vect_table: Contains all interrupt vectors and provides macros for dealing with general interrupt.

Application Layer:

Contains the main logic of the project.

## Drivers’ Documentation

### MCAL Layer

#### DIO

/*

* AUTHOR		: Bassel Yasser

* Function		: DIO_s8SETPinDir

* Description	: Set Pin Direction

* Arguments	:

* 					- enPinCopy {DIO_PINA_0...., DIO_PIND_7}

* 					- enPortDir {INPUT , OUTPUT}

* Return		:   Sint8_t

*/

Sint8_t DIO_s8SETPinDir (enu_pin enPinCopy, enu_dir enPortDir)

/*

* AUTHOR		: Bassel Yasser

* Function		: DIO_s8SETPinVal

* Description	: Set Pin Value

* Arguments	:

* 					- enPinCopy {DIO_PINA_0...., DIO_PIND_7}

* 					- enPortDir {HIGH , LOW}

* Return		:   Sint8_t

*/

Sint8_t DIO_s8SETPinVal (enu_pin enPinCopy, enu_val enPortVal)

/*

* AUTHOR		: Bassel Yasser

* Function		: DIO_s8GETPinVal

* Description	: Set Pin Value

* Arguments	:

* 					- enPinCopy {DIO_PINA_0...., DIO_PIND_7}

* 					- pu8Val address of variable that u want to save value on it

* Return		:   Sint8_t

*/

Sint8_t DIO_s8GETPinVal (enu_pin enPinCopy, Uint8_t* pu8Val)

#### Timer 0

/**

* \brief Initialize the timer with given mode

* \param u8_a_Mode

* \return en_TIMErrorState_t

*/

en_TIMErrorState_t TIM0_voidInit(en_TIMMode_t u8_a_Mode);

/**

* \brief Start the timer clock after prescaling it with given value

* \param u8_a_prescaler

* \return en_TIMErrorState_t

*/

en_TIMErrorState_t TIM0_Start(en_TIM_CLK_SELECT_t u8_a_prescaler);

/**

* \brief Function to stop timer 0

* \return void

*/

void TIM0_Stop();

/**

* \brief Set the timer to start from a certain value

* \param u8_a_FlagValue The value to start the timer from

* \return void

*/

void TIM0_SetValue(Uchar8_t u8_a_startValue);

/**

* \brief Function to get the value of the overflow flag of timer 0

* \param u8_a_FlagValue reference to a variable to store flag value *

* \return en_TIMErrorState_t

*/

en_TIMErrorState_t TIM0_GetOVF(Uchar8_t* u8_a_FlagValue);

/**

* \brief Function to clear timer 0 overflow flag

* \return void

*/

void TIM0_ClearOVF(void);

/**

* \brief Function to get the timer state (running/stopped)

* \param u8_a_State reference to a variable to store timer state

* \return en_TIMErrorState_t

*/

en_TIMErrorState_t TIM0_GetState(en_TIMState_t* u8_a_State);

/**

* \brief Function to set a function to call when the timer0

*	      Overflow Interrupt is triggered

* \param pv_a_CallbackFn reference to the function to call

* \return en_TIMErrorState_t

*/

#### EXTINT:

// EXT_INT TYPEDEFS

typedef enum EN_EXTINT_ERROR {

EXTINT_OK=0,

EXTINT_NOT_OK

}EN_EXTINT_ERROR;

typedef enum EN_Sence_Control {

LOW_LEVEL=0,

FALLING_EDGE,

RISING_EDGE,

ANY_LOGICAL_CHANGE

}EN_Sence_Control;

typedef enum EN_EXINT_NUMBER{

EXTINT0=0,

EXTINT1,

EXTINT2,

}EN_EXINT_NUMBER;

typedef enum EN_GLOBAL_INT{

DISABLE=0,

ENABLE

}EN_GLOBAL_INT;

// EXT_INT prototypes

/*

Description : This function initializes the GLOBAL_INTERRUPT

ARGS        : takes the state ( ENABLE OR DISABLE )

return      : return EXTINT_OK if the PIN initializes correctly, EXTINT_NOT_OK otherwise

*/

EN_EXTINT_ERROR SET_GLOBAL_INTERRUPT(EN_GLOBAL_INT state);

/*

Description : This function initializes the external interrupt number and it's detecting type

ARGS        : takes the EXINT_NUMBER( INT0,INT1 OR INT2) and sense control.

return      : return EXTINT_OK if the EXINT_NUMBER initializes correctly, EXTINT_NOT_OK otherwise

*/

EN_EXTINT_ERROR EXTINT_init(EN_EXINT_NUMBER INTx ,EN_Sence_Control INTxSense);

/*

Description : This function takes the external interrupt number and initialize call back function.

ARGS        : takes the EXINT_NUMBER( INT0,INT1 OR INT2) and pointer to the function we want to execute.

return      : return EXTINT_OK if the EXINT_NUMBER initializes correctly, EXTINT_NOT_OK otherwise

*/

EN_EXTINT_ERROR EXTINT_CallBack(EN_EXINT_NUMBER INTx,void(*ptrfunc)(void));

#### I2C:

/***********************************************************************************/

/*			01- i2c_init_master()									*/

/* ----------------------------------------------------------------------------------

* @func   : I2C master Initialization

* @in     : void

* @out    : void

************************************************************************************/

void i2c_init_master(void);

/***********************************************************************************/

/*			02- i2c_init_slaver()									*/

/* ----------------------------------------------------------------------------------

* @func   : I2C slave Initialization

* @in     : void

* @out    : void

************************************************************************************/

void i2c_init_slave (void);

/***********************************************************************************/

/*			03- i2c_start()									*/

/* ----------------------------------------------------------------------------------

* @func   : Send start condition

* @in     : void

* @out    : void

************************************************************************************/

void i2c_start(void);

/***********************************************************************************/

/*			04- i2c_repeated_start()									*/

/* ----------------------------------------------------------------------------------

* @func   : Send repeated start condition

* @in     : void

* @out    : void

************************************************************************************/

void i2c_repeated_start(void);

/***********************************************************************************/

/*			05- i2c_send_slave_address_with_write_req()									*/

/* ----------------------------------------------------------------------------------

* @func   		: send write request from master to slave

* @in[1]     	: slave_address

* 					- Slave address that you want to write on it

* @out       	: void

************************************************************************************/

void i2c_send_slave_address_with_write_req(Uint8_t slave_address);

/***********************************************************************************/

/*			06- i2c_send_slave_address_with_read_req()									*/

/* ----------------------------------------------------------------------------------

* @func   		: send read request from master to slave

* @in[1]     	: slave_address

* 					- Slave address that you want to read from it

* @out       	: void

************************************************************************************/

void i2c_send_slave_address_with_read_req(Uint8_t slave_address);

/***********************************************************************************/

/*			07- i2c_write_byte()									*/

/* ----------------------------------------------------------------------------------

* @func   		: Write data to slave

* @in[1]     	: byte

* 					- write data that u need to send

* @out       	: void

************************************************************************************/

void i2c_write_byte(Uint8_t byte);

/***********************************************************************************/

/*			08- i2c_read_byte()									*/

/* ----------------------------------------------------------------------------------

* @func   		: read data from slave

* @in	     	: void

* @out       	: returned data

************************************************************************************/

Uint8_t i2c_read_byte(void);

/***********************************************************************************/

/*			09- i2c_stop()									*/

/* ----------------------------------------------------------------------------------

* @func   		: Send Stop condition

* @in	     	: void

* @out       	: void

************************************************************************************/

void i2c_stop(void);

#### UART:

typedef enum EN_USART_ERROR{

USART_OK=0,

USART_NOT_OK

}EN_USART_ERROR;

/*

Name   : USART_init

Description : This function initializes USART Module with selected options in USART.cfg file.

Args   : Void (all options defined as macros).

return : Std_ReturnType (E_OK) if Module initializes Correctly, (E_NOT_OK) otherwise.

*/

EN_USART_ERROR USART_init(void);

/*

Name   : USART_sendData

Description : This function Send Data To Receiver (we can change size of data from USART.cfg file) we select 8 bits data size.

Args   : take one argument (data to be sent) must be same size as the size we select in Initialization function.

return : Std_ReturnType (E_OK) if module sent data Correctly, (E_NOT_OK) otherwise.

*/

EN_USART_ERROR USART_sendData(Uchar8_t data);

/*

Name   : USART_receiveData

Description : This function Receive Data from sender (we can change size of data from USART.cfg file) we select 8 bits data size.

Args   : void

return : data received. must be same size as the size we select in Initialization function.

*/

Uchar8_t USART_receiveData(void);

/*

Name   : USART_sendSTRING

Description : This function send array of data To Receiver. (we can change size of data from USART.cfg file) we select 8 bits data size.

Args   : pointer to the array of data to be sent

return : void

*/

void USART_sendSTRING(Uchar8_t * str);

/*

Name   : USART_receiveSTRING

Description : This function receive data from sender and store it in array.

Args   : pointer to the array to store data received in it and size of data.

return : void

*/

void USART_receiveSTRING(Uchar8_t * str,Uchar8_t size);

#### SPI:

/**

* (Author: Alaa Hisham)

* \brief Initialize the MCU as the Master

*	in SPI communication

* \return void

*/

void SPI_MasterInit(void);

/**

* (Author: Alaa Hisham)

* \brief Initialize the MCU as a slave

*	   in SPI communication

*

* \return void

*/

void SPI_SlaveInit(void);

/**

* (Author: Alaa Hisham)

* \brief Sets the value of the SPI data register

* \param u8_a_data: Desired value

* \return void

*/

void SPI_SetValue(Uchar8_t u8_a_data);

/**

* (Author: Alaa Hisham)

* \brief Exchange a letter with selected slave

* \param u8_a_character: Character to send

* \param pu8_a_receivedChar: Pointer to character to store received value

* \return en_SPI_ErrorState_t

*/

en_SPI_ErrorState_t SPI_TranscieveChar(Uchar8_t u8_a_character, Uchar8_t* pu8_a_receivedChar);

/**

* (Author: Alaa Hisham)

* \brief Exchange a letter with selected slave

* \param u8_a_character: Character to send

* \param pu8_a_receivedChar: Pointer to character to store received value

* \return en_SPI_ErrorState_t

*/

en_SPI_ErrorState_t SPI_SlaveTranscieve(Uchar8_t u8_a_character, Uchar8_t* pu8_a_receivedChar);

/**

* (Author: Alaa Hisham)

* \brief Set a notification function for the SPI Interrupt

* \param pv_a_CallbackFn: reference to the function to callback

*				when the SPI Interrupt is triggered

* \return en_SPI_ErrorState_t

*/

en_SPI_ErrorState_t SPI_SetCallback(void (*pv_a_CallbackFn)(void));

### HAL Layer:

#### Keypad

// Macros

#define R1     DIO_PINC_2

#define R2     DIO_PINC_3

#define R3     DIO_PINC_4

#define C1     DIO_PINC_5

#define C2     DIO_PINC_6

#define C3     DIO_PINC_7

// user defined datatypes

typedef enum EN_KEYPAD_BTNS

{

KEY_1=0,

KEY_2,

KEY_3,

KEY_4T,

KEY_5,

KEY_6,

KEY_7,

KEY_8,

KEY_9,

KEY_NOTHING

}EN_KEYPAD_BTNS;

// functions prototypes

/*************************************************************************************

Name : KEYPAD_init()

Description : This Function Initializes keypad pins (Rows are outputs & Columns are inputs).

ARGS : void

return : void

************************************************************************************************************/

void KEYPAD_init(void);

/*************************************************************************************************************

*

Name : KEYPAD_GetButton

Description : This Function loops over other three functions (Checks (R1,R2,R3)).

ARGS : void

return : the pressed key or Nothing pressed

************************************************************************************************************/

EN_KEYPAD_BTNS KEYPAD_GetButton(void);

/*************************************************************************************************************

*

Name : KEYPAD_checkR1 ,  KEYPAD_checkR2, KEYPAD_checkR3

Description : functions are checking the entire row if it pressed or not.

ARGS : void

return : the pressed key or Nothing pressed

************************************************************************************************************/

EN_KEYPAD_BTNS KEYPAD_checkR1(void);

EN_KEYPAD_BTNS KEYPAD_checkR2(void);

EN_KEYPAD_BTNS KEYPAD_checkR3(void);

#### HTimer:

/**

* \brief Generate Synchronous delay (busy waiting)*

* \param Copy_delayTime Desired delay

* \param Copy_timeUnit Time units (Seconds, mSeconds, uSeconds)

*

* \return en_HTIMErrorState_t

*/

en_HTIMErrorState_t TIM0_SyncDelay(Uint32_t u32_a_delay, en_timeUnits_t u8_a_timeUnit);

/**

* \brief Generates delay asynchronously

* \param u32_a_delay desired delay

* \param u8_a_timeUnit delay time units

* \param Copy_pvCallbackFn function to call when delay is complete

*

* \return en_TIMErrorState_t

*/

en_HTIMErrorState_t TIM0_AsyncDelay(Uint32_t u32_a_delay, en_timeUnits_t u8_a_timeUnit, void (*Copy_pvCallbackFn)(void));

/**

* \brief Function to end a delay asynchronously

* To Stop Async Delay: No Restrictions

* To Stop Sync Delay: should only be called in an ISR/Callback function

*

* \return void

*/

void TIM0_AsyncEndDelay();

#### HLCD

/*

* function		: HLCD_vidInit

* description 	: func to set LCD initialization

* input param 	: void

* return		: void

* */

void HLCD_vidInit(void)

/*

* function		: HLCD_vidWritecmd

* description 	: func to configure some commands on lcd

* input param 	:

* 					u8commandCopy --> take lcd cmd instructions from instruction table <https://components101.com/sites/default/files/component_datasheet/16x2%20LCD%20Datasheet.pdf>

* return		: void

* */

void HLCD_vidWritecmd(Uint8_t u8commandCopy)

/*

* function		: HLCD_vidWriteChar

* description 	: func to write char on lcd

* input param 	: u8CharCopy -> take ascii code of char   or   char address on CGROM

* return		: void

* */

void HLCD_vidWriteChar(Uint8_t u8CharCopy)

/*

* function		: HLCD_ClrDisplay

* description 	: func to clear anything on lcd

* input param 	: void

* return		: void

* */

void HLCD_ClrDisplay(void)

/*

* function		: HLCD_gotoXY

* description 	: func to determine position which char print at this position on lcd  ### NOTE : (2rows x 16coloms)

* input param 	:

* 				  row -> take row number 0 or 1

* 				  pos -> take colom number from 0 ~ 16

* return		: void

* */

void HLCD_gotoXY(Uint8_t row, Uint8_t pos)

/*

* function		: HLCD_WriteString

* description 	: func to write string on lcd

* input param 	: str --> which take string as argument

* return		: void

* */

void HLCD_WriteString(Uint8_t* str)

/*

* function		: HLCD_WriteInt

* description 	: func to write integer number on lcd

* input param 	: number --> which take number as argument

* return		: void

* */

void HLCD_WriteInt(Uint32_t number)

/*

* function		: HLCD_vidCreatCustomChar

* description 	: func to store new patterm on CGRAM

* input param 	:

* 				  pu8custom  -> take pointer to array which having LCD Custom Character Generated data ### take only 8 characters

* 				  u8Location -> determine location on CGRAM [0 ~ 8]

* return		: void

* */

void HLCD_vidCreatCustomChar(Uint8_t* pu8custom, Uint8_t u8Location)

/**

* (Author: Alaa Hisham)

* \brief Display floating point number on LCD

*

* \param f32_a_number: number to display

* \return void

*/

void HLCD_DisplayFloat(float32_t f32_a_number);

#### Buzzer

/**

* \brief Initialize buzzer pin as output

* \param pst_a_buzzer reference to buzzer

* \return void

*/

void BUZ_Init(st_Buzzer_t* pst_a_buzzer);

/**

* \brief Turn the buzzer on/off

* \param pst_a_buzzer reference to buzzer

* \param u16_a_state BUZ_ON (or) BUZ_OFF

* \return en_BuzzerErrorState_t

*/

en_BuzzerErrorState_t BUZ_SetState(st_Buzzer_t* pst_a_buzzer, en_BuzzerState_t en_a_state);

#### Button

/*

* AUTHOR			: Bassel Yasser Mahmoud

* FUNCTION			: HButton_Init

* DESCRIPTION		: Initialize specified pin as input and pull up

* RETURN			: enu_buttonError_t {BUTTON_NOK, BUTTON_OK}

*/

enu_buttonError_t HButton_Init(enu_pin en_pinx);

/*

* AUTHOR			: Bassel Yasser Mahmoud

* FUNCTION			: HButton_ExtIntInit

* DESCRIPTION		: Initialize specified as pull up for external interrupt

* RETURN			: enu_buttonError_t {BUTTON_NOK, BUTTON_OK}

*/

enu_buttonError_t HButton_ExtIntInit(enu_pin en_pinx);

/*

* AUTHOR			: Bassel Yasser Mahmoud

* FUNCTION			: HButton_getPinVal

* DESCRIPTION		: Get pin status if it is high or low

* RETURN			: enu_buttonError_t {BUTTON_NOK, BUTTON_OK}

*/

enu_buttonError_t HButton_getPinVal(enu_pin en_pinx,  Uint8_t* pu8_refVal );

*

Description : This function initializes the external interrupt number and it's detecting type and initialize call back function.

ARGS        : takes the EXINT_NUMBER( INT0,INT1 OR INT2) and sense control and and pointer to the function we want to execute when interrupt occurs.

return      : return EXTINT_OK if the EXINT_NUMBER initializes correctly, EXTINT_NOT_OK otherwise

*/

EN_EXTINT_ERROR H_EXTINT_create(EN_EXINT_NUMBER INTx ,EN_Sence_Control INTxSense,void(*ptrfunc)(void));

#### HEXT_INT

*

Description : This function initializes the external interrupt number and it's detecting type and initialize call back function.

ARGS        : takes the EXINT_NUMBER( INT0,INT1 OR INT2) and sense control and and pointer to the function we want to execute when interrupt occurs.

return      : return EXTINT_OK if the EXINT_NUMBER initializes correctly, EXTINT_NOT_OK otherwise

*/

EN_EXTINT_ERROR H_EXTINT_create(EN_EXINT_NUMBER INTx ,EN_Sence_Control INTxSense,void(*ptrfunc)(void));

#### HSPI

/**

* (Author: Alaa Hisham)

* \brief Initialize the MCU as the Master in SPI communication

* \return void

*/

void HSPI_MasterInit(void);

/**

* (Author: Alaa Hisham)

* \brief Initialize the MCU as a slave in SPI communication

* \return void

*/

void HSPI_SlaveInit(void);

/**

* (Author: Alaa Hisham)

* \brief Function to send a single character

* \param u8_a_character character to send

* \return void

*/

void HSPI_SendChar(Uchar8_t u8_a_character);

/**

* \brief Receive data in the given buffer

*

* \param pu8_a_data: reference to buffer to store received data

* \param u8_a_DataSize: Size (length) of data to receive

*

* \return void

*/

void HSPI_ReceiveData(Uchar8_t *pu8_a_data, Uchar8_t u8_a_DataSize);

/**

* \brief Send given data byte by byte to selected slave

*		  and receive data in exchange into given array

* \param pu8_a_TxDataArr: Reference to array of data to be transmitted

* \param pu8_a_RxDataArr: Reference to array to store received data

* \param u8_a_DataLen:	   Length of data to exchange (in bytes)

*					       (Must be less than or equal SPI_BUFFER_SIZE)

*

* \return en_HSPI_ErrorState_t

*/

en_HSPI_ErrorState_t HSPI_ExchangeData(Uchar8_t* pu8_a_TxDataArr, Uchar8_t* pu8_a_RxDataArr, Uchar8_t u8_a_DataLen);

/**

* (Author: Alaa Hisham)

* \brief Request to send data to the master

* \param u8_a_data: data to send

* \return void

*/

void HSPI_SlaveRequest(Uchar8_t* pu8_a_dataPtr, Uchar8_t u8_a_DataSize);

#### EEPROM:

/** AUTHOR : Sherif Ashraf Khadr

* \brief : This Function Just Call To Initialize I2C as Master

*

* \param : Void

*

* \return void

*/

void eeprom_init(void)

/** AUTHOR : Sherif Ashraf Khadr

* \brief : This Function Call To Make The Sequence Of I2C Frame To Write Byte On A Device

*

* \param : Uint16_t address : This Is Device Address

*		 : Uchar8_t data : This Is Data That Will Be Write

*

* \return void

*/

void eeprom_write_byte(Uint16_t address, Uchar8_t data)

/** AUTHOR : Sherif Ashraf Khadr

* \brief : This Function Call To Make The Sequence Of I2C Frame To Read Byte On A Device

*

* \param : Uint16_t address : This Is Device Address

*

* \return Uchar8_t : Function Will Return Uchar8_t Contain The Data

*/

Uchar8_t eeprom_read_byte(Uint16_t address)

/** AUTHOR : Sherif Ashraf Khadr

* \brief : This Function Call When You Need To Write A String In The EEPROM

*

* \param : Uint16_t address : This Is Device Address

*		 : Uchar8_t *str	: This Pointer Will Store The Address Of The Array Of The Chars

* \return  Void

*/

void eeprom_write_string(Uint16_t Copy_u8Address, const Uchar8_t* str)

/** AUTHOR : Sherif Ashraf Khadr

* \brief : This Function Call When You Need To Read A String From The EEPROM

*

* \param : Uint16_t address : This Is Device Address

*		 : Uchar8_t *str	: This Pointer Will Store The Address Of The Array Of The Chars That Will Return String In It

* \return  Void

*/

void eeprom_read_string(Uint16_t Copy_u8Address, Uchar8_t* str)

### Card_Database Layer (CARD MCU)

/*

* AUTHOR			: Bassel Yasser Mahmoud

* FUNCTION			: APP_terminalPinGet

* DESCRIPTION		: Get pin from User within terminal and doing some validation

* RETURN			: en_terminalPinGetStatus_t {PINGET_NOK or PINGET_OK}

*/

en_terminalPinGetStatus_t APP_terminalPinGet(Uchar8_t* arr);

/*

* AUTHOR			: Sharpel

* FUNCTION			: APP_terminalPanGet

* DESCRIPTION		: Get pan from User within terminal and doing some validation

* RETURN			: en_terminalPanGetStatus_t {PANGET_NOK or PANGET_OK}

*/

en_terminalPanGetStatus_t APP_terminalPanGet(Uchar8_t* arr);

/*

* FUNCTION			: SaveCardData

* DESCRIPTION		: Saving PAN and PIN in EEPROM

* RETURN			: EN_TerminalDataState {DATA_SAVED, DATA_NSAVED, DATA_READ, DATA_NREAD}

*/

EN_TerminalDataState SaveCardData(Uchar8_t *CardPan,Uchar8_t *CardPin);

/*

* FUNCTION			: ReadCardData

* DESCRIPTION		: Reading PAN and PIN from EEPROM

* RETURN			: EN_TerminalDataState {DATA_SAVED, DATA_NSAVED, DATA_READ, DATA_NREAD}

*/

EN_TerminalDataState ReadCardData(Uchar8_t *CardPan,Uchar8_t *CardPin);

/*

* FUNCTION			: CARD_MatchPINs

* DESCRIPTION		: Validate if PIN no and Confirmed PIN no is Matched or not

* RETURN			: en_CardPinMatchError_t {	PIN_Match_NOK, PIN_Match_OK}

*/

en_CardPinMatchError_t CARD_MatchPINs();

### ATM_MODULE Layer (ATM MCU)

#### ATM:

/**

* \brief Displays welcome routine

*

* \return void

*/

void Welcome(void);

/**

* (Author: Sherif Ashraf)

* \brief Check if the pin user enters is the same as the cardholder's pin

* \param pinFromAtm   : reference to pin entered by user

* \param pinFromServer: reference to pin received from card

* \return EN_PinState

*/

EN_PinState PIN_checkPinMatching(Uchar8_t *pinFromAtm,Uchar8_t *pinFromServer);

/*

* AUTHOR			: Sharpel

* FUNCTION			: Get_pin

* DESCRIPTION		: get pin from user (on the atm )

* ARGS		        : pointer to array (the size of array must be 5 or more) to store entered pin by user

* RETURN			: PIN_OK if user enters 4 numbers , PIN_NOT_OK otherwise

*/

EN_PinState Get_pin(Uchar8_t *enteredpin);

/**

* (Author: Sherif Ashraf)

* \brief Locks the system and sound the buzzer

* \param pst_a_buzzer: reference to the buzzer

* \return en_BuzzerErrorState_t

*/

en_BuzzerErrorState_t deinitAtm(st_Buzzer_t* pst_a_buzzer);

/**

* (Author: Alaa Hisham)

* \brief Get the card pan and pin

* \param pu8_a_pan: reference to buffer to receive pan from card

* \param pu8_a_pin: reference to buffer to receive pin from card

* \return EN_PinState

*/

EN_PinState ATM_GetCardData(Uchar8_t *pu8_a_pan, Uchar8_t *pu8_a_pin);

/*

* AUTHOR			: Sharpel

* FUNCTION			: get_amount_left

* DESCRIPTION		: get amount from user ( on the atm )

* ARGS		        : pointer to array (the size of array must be 8 or more and equal "0000.00" initial value) to store entered pin by user

* RETURN			: void

*/

void get_amount_left (Uchar8_t * amount);

/*

* AUTHOR			: Bassel Yasser

* FUNCTION			: EXTINT_FUNC

* DESCRIPTION		: when timer 2 ISR is fire it changes the state of (Enter or Zero)

* ARGS		        : void

* RETURN			: void

*/

void EXTINT_FUNC(void);

/**

* (Author: Alaa Hisham)

* \brief Carries out the routine for approved card

* \param f32_a_NewBalance: the balance to display after transaction

* \return void

*/

void ATM_ApprovedCard(float32_t f32_a_NewBalance);

#### Database_check:

/*

* FUNCTION			: isValidPanAccount

* DESCRIPTION		: Check If PAN No. Valid or not

* RETURN			: EN_dataError_t {APPROVED ,FRAUD_CARD,CARD_STOLEN,EXCEED_MAX_DAILY_AMOUNT,INSUFFICIENT_FUND, DATA_ERROR}

*/

EN_dataError_t isValidPanAccount(Uchar8_t * pan);

/*

* FUNCTION			: isRunningAccount

* DESCRIPTION		: Checking if card stolen or not

* RETURN			: EN_dataError_t {APPROVED ,FRAUD_CARD,CARD_STOLEN,EXCEED_MAX_DAILY_AMOUNT,INSUFFICIENT_FUND, DATA_ERROR}

*/

EN_dataError_t isRunningAccount(Uchar8_t * pan);

/*

* FUNCTION			: isValidAccountAmount

* DESCRIPTION		: Checking if there is INSUFFICIENT_FUND or not

* RETURN			: EN_dataError_t {APPROVED ,FRAUD_CARD,CARD_STOLEN,EXCEED_MAX_DAILY_AMOUNT,INSUFFICIENT_FUND, DATA_ERROR}

*/

EN_dataError_t isValidAccountAmount(Uchar8_t * pan,Uchar8_t * amount,float32_t *newAmount);

/*

* FUNCTION			: isBelowMaxDailyAmount

* DESCRIPTION		: Checking if transfered money is below limited daily amount or not

* RETURN			: EN_dataError_t {APPROVED ,FRAUD_CARD,CARD_STOLEN,EXCEED_MAX_DAILY_AMOUNT,INSUFFICIENT_FUND, DATA_ERROR}

*/

EN_dataError_t isBelowMaxDailyAmount(Uchar8_t * amount);

/*

* FUNCTION			: DATABASE_checking

* DESCRIPTION		: Card Database checking

* RETURN			: EN_dataError_t {APPROVED ,FRAUD_CARD,CARD_STOLEN,EXCEED_MAX_DAILY_AMOUNT,INSUFFICIENT_FUND, DATA_ERROR}

*/

EN_dataError_t DATABASE_checking (Uchar8_t * pan,Uchar8_t * amount,float32_t *newAmount);

### Application Layer:

#### Card MCU

/**

* \brief Initializations of all used peripherals

* \return void

*/

void APP_Init(void);

/**

* \brief The main logic of the Card

* \return void

*/

void APP_Start(void);

#### ATM MCU

/**

* \brief Initializations of all used peripherals

* \return void

*/

void APP_Init(void);

/**

* \brief The main logic of the ATM

* \return void

*/

void APP_Start(void);

# Low Level Design

### MCAL Layer:

#### DIO

Sint8_t DIO_s8SETPinDir (enu_pin enPinCopy, enu_dir enPortDir)

Figure 2 DIO_s8SETPinDir Flow Chart

Sint8_t DIO_s8SETPinVal (enu_pin enPinCopy, enu_val enPortVal)

Figure 3 DIO_s8SETPinVal Flow chart

Sint8_t DIO_s8GETPinVal (enu_pin enPinCopy, Uint8_t* pu8Val)

Figure 4 DIO_s8GETPinVal Flow Chart

#### Timer:

TIM0_Init

Figure 5 TIM0_Init Flow Chart

TIM0_Start

Figure 6  TIM0_Start Flow Chart

TIM0_Stop

Figure 7 TIM0_Stop Flow Chart

Figure 8 TIM0 remaining Flow Charts

#### EXTINT:

#### UART:

#### SPI:

#### I2C:

void i2c_init_master(void);

void i2c_start(void);

void i2c_repeated_start(void);

void i2c_send_slave_address_with_write_req(Uint8_t slave_address);

void i2c_send_slave_address_with_read_req(Uint8_t slave_address);

void i2c_write_byte(Uint8_t byte);

Uint8_t i2c_read_byte(void);

void i2c_stop(void);

### HAL Layer

#### HTimer0

HTIM0_SyncDelay

Figure 15 HTIM0_SyncDelay Flow Chart

Figure 16 HTIM0_AsyncDelay and EndDelay

#### HSPI

#### LCD

Figure 17 HLCD_vidInit Flow Chart

void HLCD_vidInit(void)

void HLCD_vidWritecmd(Uint8_t u8commandCopy)

Figure 18 HLCD_vidWritecmd Flow Chart

void HLCD_vidWriteChar(Uint8_t u8CharCopy)

Figure 19 HLCD_vidWriteChar Flow Chart

Figure 20 HLCD_ClrDisplay Flow Chart

void HLCD_ClrDisplay(void)

void HLCD_gotoXY(Uint8_t row, Uint8_t pos)

Figure 21 HLCD_gotoXY Flow Chart

void HLCD_WriteString(Uint8_t* str)

Figure 22 HLCD_WriteString Flow Chart

void HLCD_WriteInt(Uint32_t number)

Figure 23 HLCD_WriteInt Flow Chart

void HLCD_vidCreatCustomChar(Uint8_t* pu8custom, Uint8_t u8Location)

Figure 24 HLCD_vidCreatCustomChar Flow Chart

void HLCD_DisplayFloat(float32_t f32_a_number);

#### Keypad

Figure 25 KEYPAD_Init Flow Chart

Figure 26 KEYPAD_CheckRx Flow Chart

Figure 27 GetButton Flow Chart

#### Buzzer

BUZ_SetState

Figure 28 Buzzer Init & SetState Flow Charts

BUZ_Init

#### HEXTINT:

#### Button:

enu_buttonError_t HButton_Init(enu_pin en_pinx);

enu_buttonError_t HButton_ExtIntInit(enu_pin en_pinx);

enu_buttonError_t HButton_getPinVal(enu_pin en_pinx,  Uint8_t* pu8_refVal );

#### EEPROM

void eeprom_init(void)

void eeprom_write_byte(Uint16_t address, Uchar8_t data)

Uchar8_t eeprom_read_byte(Uint16_t address)

void eeprom_write_string(Uint16_t Copy_u8Address, const Uchar8_t* str)

void eeprom_read_string(Uint16_t Copy_u8Address, Uchar8_t* str)

### Card_Database Layer (CARD MCU)

en_terminalPinGetStatus_t APP_terminalPinGet(Uchar8_t* arr);

en_terminalPanGetStatus_t APP_terminalPanGet(Uchar8_t* arr);

SaveCardData

ReadCardData

#### Database_check:

EN_dataError_t isValidPanAccount(Uchar8_t * pan);

EN_dataError_t isRunningAccount(Uchar8_t * pan);

EN_dataError_t isValidAccountAmount(Uchar8_t * pan,Uchar8_t * amount,float32_t *newAmount);

EN_dataError_t isBelowMaxDailyAmount(Uchar8_t * amount);

EN_dataError_t DATABASE_checking (Uchar8_t * pan,Uchar8_t * amount,float32_t *newAmount);

#### ATM Module

PIN_checkPinMatching

deinitAtm

### Application Layer:

### Card App

Figure 30 Card App. APIs Flow Charts

### ATM App